Release Notes

| Program: AV220C2+ | Revision Number: 5.72.1113’12 Replaced Revision: 5.72.1012’11 Creation Date: 11/13/2012 |  |
| --- | --- | --- |
| Author or Designer: Pao |  |  |
| What Changed? |  |  |
| What has been Fixed? | Avscan_n.dll - Bug fixed for cleaning print header mode. - Support image encoder library from 3rd party. - Fixed the pixel boundary mistake in memory transfer. - Fixed the incorrect image after processing in color match photo mode. - Support to export front side MICR only while duplex scanning. - Fix the function of cleaning print head is not working. - Color matching is disable on Windows 2000. Advance.dll - Update Endorser PrintCounter via get message from outside. - Add Paperweight. - Add GrayPowerSavingControl into ADVUI.INI, to determine disable or enable power - Add Profile parameter into default.av2, to determine user default profile. - Paper protection supported. - Modify that image counts option will enable while flatbed not support batchscan. - Modify that Scan Area will show "None" while user adjust X/Y offset in Paper tab. - Read ShowDeskew parameter from ADVUI.INI, to determine display Deskew option in Paper tab. - Color matching is disable on Windows 2000. AV220C2+.ds - Reset the current transfer type in DAT_IMAGEINFO. - New capability ICAP_MAGNIFICATION for image magnification processing. - Send the pointer of Scanner Ability to Scan interface. - Change the minimum power saving time from 1 to 0. - New custom capabilities CAP_POWEROFF and CAP_PAPERWEIGHT. - Fixed to send power off time function if scanner unsupported. - Simplex sheetfed scanner return incorrect status in EnableDS while feeder empty. - Support Windows 8/Windows Server 2012 OS version determination. - New definition capability CAP_CLEANPRINTHEAD for cleaning print head. - Reset the clean print head setting after one page done. Twdsm_n.dll - Bug fixed to generate the wrong setting file on Windows XP 64-bits without Administrator permission. AVIP.dll - Watermark processing enhancement. - Fixed memory leaks. - Fixed the unreadable text in Japanese OS - Fixed the transparency setting failure of watermark in B&W.. |
| Known Defects |  |  |
| Required Files And Revision Level | File Name | Revision |
|  | Avscan_n.dll | 1.3.2.33 |
|  | Advance.dll | 1.3.0.85 |
|  | AV220C2+.ds | 2.1.1.42 |
|  | AVglowing.dll | 1.0.49.0 |
|  | AVBarcode.dll | 1.0.0.5 |
|  | Twdsm_n.dll | 1.2.2.3 |
|  | AVIP.dll | 0.0.0.47 |
|  | AV220C2Plus.dll | 0.9.2009.618 |
|  | SmartImage.dll | 1.0.144.0 |
|  | Av2PDF.dll | 1.0.0.6 |
|  | AVBlank.dll | 0.1.0.3 |
|  | AvCropEx.dll | 0.5.10.0 |
|  | AVJpegEncoder.dll | 1.1.0.4 |
|  | AVScanDoubleSizeLib.dll | 1.3.0.1 |
|  | Cmatch.dll | 4.3.9.3 |
|  | ColorDropOut.dll | 2005/08/26 |
|  | fpcreator_img.dll | 1.0.0.617 |
|  | Gray_quality.dll | 1.0.0.5 |
|  | LDyIp.dll | 2.0.1.11 |
| Details: |  |  |